Dear Customer

Custom Tuning

VAT Reg. 917 740 512

Unit 5 Rugby Business Park

Rugby Street, Hull, HU3 4RB

United Kingdom

+44 (0) 1482 224433

Thank you for purchasing your new exclusive dsci exhaust system, this is provided with a full fitting kit and should be done using the usual safety precautions. Once fitted the exhaust will produce thick white smoke for a few minutes, this is correct as it is excess cutting oil being burnt away. The exhaust will have a slightly deeper tone until some small deposits of carbon have been allowed to build up and soften the sound. Make sure you clean the rear in line resonators before applying the heat proof metal stickers as per the picture on the reverse. Also check for movement after initial installation.

The centre sections should be joined with high grade silicone, add a small bead to both surfaces first, allow this to partially dry for 15 minutes then apply another thicker layer and clamp the 2 surfaces together and it’s ready to drive straight away. Ensure you position your exhaust tips when the system is hot to avoid movement and a burnt bumper, then re check and if necessary re-position after first run as some expansion can occur.

All the best

Carl Balmforth

Technical Manager

Dreamscience Automotive ltd. Company Reg. 05573002, web; www.dreamscience.eu , email; sales@dreamscience.eu